SmartDevice-Handheld BLE ICD

Version 0.13 - DA14531 P531 Handheld as-built interface

| Item | Value |
| --- | --- |
| Applies to | DA14531MOD-based P531 handheld wand and companion SmartDevice application |
| Production BLE name | P531-Handheld |
| Recovery BLE name | P531-Recovery |
| Protocol version | 0x01 |
| Updated for | DEVICE_INFO_SET, PPFD calibration index 6, averaging limits, six wavelength channels, recovery/update flow, and flash diagnostic logging |

As-built scope: this document describes the current target branch implementation. Reserved commands/events from earlier drafts are omitted unless implemented. Image update regression is handled separately from normal ICD regression because full image transfer is slow.

# 1. Roles and Transport

SmartDevice is the master. It initiates all requests, owns sequencing, retries, and user-facing workflow.

Handheld is the BLE peripheral/slave. It responds using the same OPC with FLAGS.b2=1 and echoes SEQ.

Primary service UUID: 9E0F0001-5310-4C1F-AAAA-BBBBCCCCDD01.

ICD RX/control characteristic UUID: 9E0F0002-5310-4C1F-AAAA-BBBBCCCCDD01. SmartDevice writes request frames here.

ICD TX/notify characteristic UUID: 9E0F0003-5310-4C1F-AAAA-BBBBCCCCDD01. Handheld sends responses/notifications here.

Default ATT_MTU is 23. The implementation fragments complete ICD frames over GATT values as needed; the CRC belongs to the complete ICD frame.

| Advertised state | BLE local name | Meaning | SmartDevice expectation |
| --- | --- | --- | --- |
| Production application | P531-Handheld | Normal measurement/calibration firmware is running. | Use full command set in this document. |
| Recovery application | P531-Recovery | Minimal recovery firmware is running and ready for image update. | Use PING, GET_INFO, IMAGE_BEGIN, IMAGE_DATA, IMAGE_END, and REBOOT. Other opcodes may return UNKNOWN_OPCODE. |

# 2. Frame Format

| Field | Offset | Size | Type | Description |
| --- | --- | --- | --- | --- |
| VER | 0 | 1 | u8 | Protocol version. Current value: 0x01. |
| OPC | 1 | 1 | u8 | Command opcode or event opcode. |
| FLAGS | 2 | 1 | u8 | b0 FragStart reserved, b1 FragCont reserved, b2 Resp, b3 AckReq reserved. |
| SEQ | 3 | 1 | u8 | SmartDevice-selected sequence value. Responses echo this value. |
| LEN | 4 | 2 | u16le | Payload length in bytes. Response payloads include RET as byte 0. |
| PAYLOAD | 6 | LEN | bytes | Command-specific payload. Requests do not include RET. Responses always begin with RET. |
| CRC16 | 6+LEN | 2 | u16le | CRC-16/X25 over VER..PAYLOAD. |

| RET | Name | Meaning |
| --- | --- | --- |
| 0x00 | OK | Command accepted/completed. |
| 0x01 | UNKNOWN_OPCODE | Opcode not implemented in the current advertised state. |
| 0x02 | BAD_PARAM | Payload length or field value invalid. |
| 0x03 | BUSY | Device busy. |
| 0x04 | NOT_READY | Required state is missing, for example peripheral power is off before START_MEAS. |
| 0x05 | DENIED | Rejected by security/policy. |
| 0x06 | CRC_FAIL | Frame CRC mismatch. |
| 0x07 | TIMEOUT | Operation timeout. |
| 0x08 | TOO_LONG | Payload too long. |
| 0x09 | UNSUPPORTED | Valid field not supported by this build. |
| 0x0A | LOW_POWER_GUARD | Rejected by low-power guard. |

# 3. Production Command Set

| OPC | Name | Request | Response | Notes |
| --- | --- | --- | --- | --- |
| 0x01 | PING | - | RET, UPTIME_MS:u32 | Liveness check. |
| 0x02 | GET_INFO | - | RET, FW_VER[3], HW_REV:u8, SERIAL:u32, BUILD_ID:u32 | SERIAL/HW_REV come from provisioned flash identity. |
| 0x03 | GET_STATUS | - | RET, status struct | See Section 4. |
| 0x04 | SET_TIME | EPOCH_S:u32 | RET | Sets handheld time base used by status/log timestamps. |
| 0x05 | SET_RED_LED | MODE:u8, PERIOD_100MS:u16, DUTY_PCT:u8 | RET | MODE: 0 off, 1 on, 2 blink. |
| 0x06 | SET_GREEN_LED | MODE:u8, PERIOD_100MS:u16, DUTY_PCT:u8 | RET | MODE: 0 off, 1 on, 2 blink. |
| 0x07 | SET_PERIPH_POWER | EN:u8 (0/1) | RET | Must be ON before START_MEAS. Logs power transitions. |
| 0x08 | REBOOT | - | RET then disconnect/reset | Commits pending verified image before reset. |
| 0x09 | ENTER_DEEP_SLEEP | - | RET then sleep | Target-board wake validation deferred; dev-board wake is not authoritative. |
| 0x0A | DEVICE_INFO_SET | HW_REV:u8, SERIAL:u32 | RET | Manufacturing/calibration provisioning. Rejects SERIAL=0xFFFFFFFF. |
| 0x10 | START_MEAS | MODE:u8, COUNT:u16, INTERVAL_MS:u16, MEAS_AVG:u8, FLAGS:u8 | RET, measurement frame | Only MODE=0 ONESHOT implemented. Requires peripheral power ON. |
| 0x14 | LOG_GET | START_IDX:u16, MAX_ITEMS:u16 | RET, N:u16, N records | Returns up to 5 compact log records per request. |
| 0x15 | LOG_CLEAR | - | RET | Erases the log sector and resets log count to 0. |
| 0x22 | CAL_GET | W_IDX:u8 | RET, CAL_VER, CAL_FLAGS, N, CAL_REC | W_IDX 0..5 LEDs, W_IDX 6 PPFD. |
| 0x23 | CAL_SET | CAL_REC (11 bytes) | RET | NVM commit. Current local returns: 0 OK, 1 bad len/not found, 2 flash/bounds error. |
| 0x40 | IMAGE_BEGIN | Preferred compact: TYPE:u8, SIZE:u32, VER[3], CRC32:u32. Legacy accepted: TYPE:u8, COMPRESS:u8, SIZE:u32, VER[3], CRC32:u32 | RET, WINDOW:u16, SLOT:u8 | Starts staged production image upload. |
| 0x41 | IMAGE_DATA | OFFSET:u16, BYTES:u8, DATA[BYTES] | RET | Chunk data. BYTES <= available ICD payload and <=255. |
| 0x42 | IMAGE_END | - | RET, VERIFY:u8 | VERIFY: 0 OK, 1 size mismatch, 2 CRC mismatch, 3 other. REBOOT required to run new image. |
| 0x51 | LED_CUR_SET | W_IDX:u8, I_LED_UA:u16 | RET | Only W_IDX 0..5. W_IDX 6 is PPFD and is rejected. |
| 0x52 | LED_CUR_GET | W_IDX:u8 | RET, W_IDX:u8, I_LED_UA:u16 | Reads stored current for physical LED index. |

## 3.1 Device Identity Provisioning

GET_INFO is the readback path for firmware version, hardware revision, serial number, and build ID.

DEVICE_INFO_SET writes HW_REV and SERIAL into nonvolatile flash-backed identity storage used by GET_INFO.

FW_VER and BUILD_ID are firmware-derived and are not settable by the SmartDevice.

Blank/unprovisioned devices read back SERIAL=0 and HW_REV=0. SERIAL=0xFFFFFFFF is reserved for erased flash and rejected.

Recommended production flow: provision identity during calibration, immediately verify with GET_INFO, and fail units that still report SERIAL=0 unless intentionally unprovisioned.

# 4. Status and Measurement Payloads

| GET_STATUS field | Type | Units | Description |
| --- | --- | --- | --- |
| UPTIME_MS | u32 | ms | Time since application boot. |
| BATT_MV | u16 | mV | Battery voltage. |
| PERIPH_PWR | u8 | bool | Peripheral power state. |
| SENSOR_FLAGS | u16 | bitmap | Current sensor presence/status bitmap. |
| FAULT_FLAGS | u16 | bitmap | Current fault bitmap. |
| RSSI_DBM | i8 | dBm | Latest connection RSSI. |
| LAST_RESET | u8 | enum | Reset reason from platform status. |
| HANDHELD_TIME_S | u32 | s | Unix time if SET_TIME has been called; otherwise uptime-derived seconds. |

| START_MEAS request field | Type | Rule |
| --- | --- | --- |
| MODE | u8 | Only 0=ONESHOT implemented. Other values return BAD_PARAM. |
| COUNT | u16 | Set to 1 for current ONESHOT flow. Parsed for payload shape compatibility. |
| INTERVAL_MS | u16 | Must be >=200. Used for host pacing compatibility. |
| MEAS_AVG | u8 | Allowed: 0, 1, 2, 4, 8, 16. 0 is treated as 1. Averaging applies only to ADC-derived fields. |
| FLAGS | u8 | Reserved; set 0. |

| Measurement response field | Type | Units | Description |
| --- | --- | --- | --- |
| RET | u8 | - | 0 OK, or error. |
| MEAS_ID | u16 | - | Monotonic measurement ID. |
| HANDHELD_TIME_S | u32 | s | Current handheld time. Firmware member name is ts_ms, but value is seconds. |
| SENSORS | u16 | bitmap | Sensor snapshot. |
| AMB_TEMP_CC | i16 | 0.01 C | Ambient temperature from HDC2010. |
| RH_CPERCENT | u16 | 0.01 %RH | Relative humidity from HDC2010. |
| PPFD_RAW_MV | u16 | mV | PPFD photodiode ADC millivolts. SmartDevice maps mV to PPFD using CAL W_IDX=6. |
| LEAF_TEMP_CC | i16 | 0.01 C | Leaf/object temperature from MLX90632. |
| ACCEL_X/Y/Z_MG | 3 x i16 | mg | MXC4005 axes. |
| ADC_V_MV[12] | 12 x u16 | mV | Physical LED reflectance channels use indices 0..5; remaining entries are reserved/zero. |
| BATT_MV | u16 | mV | Battery voltage. |
| ERRORS | u16 | bitmap | b0 indicates target hardware measurement backend failure. |

START_MEAS state rule: SmartDevice must send SET_PERIPH_POWER=1 before START_MEAS. If peripheral power is off, START_MEAS returns NOT_READY and records a MEAS_REJECT_POWER diagnostic log entry.

# 5. Wavelengths, PPFD, Calibration, and LED Currents

| W_IDX | Physical channel | Measurement response | Calibration use |
| --- | --- | --- | --- |
| 0 | 531 nm LED | ADC_V_MV[0] | Reflectance CAL_REC. |
| 1 | 570 nm LED | ADC_V_MV[1] | Reflectance CAL_REC. |
| 2 | 705 nm LED | ADC_V_MV[2] | Reflectance CAL_REC. |
| 3 | 750 nm LED | ADC_V_MV[3] | Reflectance CAL_REC. |
| 4 | 830 nm LED | ADC_V_MV[4] | Reflectance CAL_REC. |
| 5 | 970 nm LED | ADC_V_MV[5] | Reflectance CAL_REC. |
| 6 | PPFD photodiode pseudo-index | PPFD_RAW_MV field, not ADC_V_MV[6] | PPFD two-point mV-to-PPFD calibration. LED_CUR_SET rejects this index. |

| CAL_REC field | Type | LED W_IDX 0..5 | PPFD W_IDX 6 |
| --- | --- | --- | --- |
| W_IDX | u8 | 0..5 | 6 |
| R1 | u16 | Reflectivity point 1 in Q1.15 | PPFD point 1, PPFD x10 |
| V1_MV | u16 | Measured photodiode mV at R1 | Measured PPFD photodiode mV at point 1 |
| R2 | u16 | Reflectivity point 2 in Q1.15 | PPFD point 2, PPFD x10 |
| V2_MV | u16 | Measured photodiode mV at R2 | Measured PPFD photodiode mV at point 2 |
| COV_MV | u16 | Cover/window diagnostic mV | Reserved; set 0 |

CAL_GET request payload is W_IDX:u8. Response payload is RET, CAL_VER:u8, CAL_FLAGS:u16, N:u8, then one 11-byte CAL_REC when N=1.

CAL_SET request payload is exactly one 11-byte CAL_REC.

LED_CUR_SET and LED_CUR_GET apply only to physical LED indices 0..5. LED current is stored in microamps.

For all six LEDs, photocurrent is read from the single LED photodiode ADC channel; there is no per-LED photodiode mapping in the ICD.

# 6. Diagnostic Flash Log

The production firmware keeps an append-only diagnostic log in flash for external visibility into issues.

Record size is 8 bytes: TS_S:u32, CODE:u8, ARG:u16, RESERVED:u8.

Current capacity is 512 records in the first 4 KB log sector. Additional reserved log flash is not used by the current logger.

LOG_GET returns a window of compact 7-byte records: TS_S:u32, CODE:u8, ARG:u16. RESERVED is not transmitted.

Current firmware caps each LOG_GET response to 5 records to keep response payload bounded.

LOG_CLEAR erases the log sector and resets count to zero.

| Code | Name | ARG meaning |
| --- | --- | --- |
| 0x13 | MEAS_HW_FAIL | stage<<8 ¦ detail |
| 0x14 | MEAS_REJECT_PARAM | mode<<8 ¦ avg |
| 0x15 | MEAS_REJECT_POWER | avg |
| 0x23 | IMAGE_BEGIN_FAIL | return/status |
| 0x25 | IMAGE_DATA_FAIL | return/status |
| 0x27 | IMG_FLASH_ID_BAD | flash device id |
| 0x28 | IMG_BEGIN_BAD | bad length/size/end low word |
| 0x29 | IMG_BOOT_HDR_WRITE_FAIL | written bytes |
| 0x2A | IMG_DATA_WRITE_MISMATCH | actual bytes |
| 0x2B | IMG_END_SIZE_MISMATCH | expected size low word |
| 0x2D | IMG_LAYOUT_SANITY_FAIL | reason |
| 0x2E | IMG_FLASH_NOT_READY | stage |
| 0x31 | CAL_SAVE_ERR | W_IDX |
| 0x33 | LED_CUR_SAVE_ERR | W_IDX |
| 0x34..0x38 | CAL backend errors | backend-specific |
| 0x39 | DEVICE_INFO_SET | HW_REV |
| 0x41 | LOG_CLEAR_OK | reserved |
| 0x42 | LOG_CLEAR_ERR | erase status |
| 0x50 | ICD_TX_OVERSIZE | OPC |
| 0x51 | ICD_LEN_MISMATCH | received length |
| 0x52 | ICD_UNKNOWN_OPCODE | OPC |
| 0x53 | PERIPH_POWER | 0 off / 1 on |
| 0x54 | REBOOT_CMD | 0 |
| 0x55 | SLEEP_CMD | 0 |
| 0x60 | BCB_FLASH_NOT_READY | 0 |
| 0x61 | BCB_CRC_BAD | 0 |
| 0x62 | BCB_WRITE_SHORT | written bytes |

# 7. Firmware Update and Recovery

| Flash region | Offset | Size | Purpose |
| --- | --- | --- | --- |
| Boot manager | 0x00000 | 0x03000 | Stage-1 boot manager, DA14531 bootable format. |
| Slot A | 0x03000 | 0x0B000 | Production application slot. IMAGE_* writes here. |
| Slot B | 0x0E000 | 0x0C000 | Recovery application slot. |
| Logs | 0x1A000 | 0x04000 | Reserved log region; current logger uses first 4 KB. |
| BCB | 0x1E000 | 0x01000 | Boot-control block. |
| CAL/identity | 0x1F000 | 0x01000 | Calibration, LED currents, hardware revision, serial. |

SmartDevice connects to P531-Handheld and confirms identity with GET_INFO.

SmartDevice sends IMAGE_BEGIN using the preferred compact 12-byte payload with raw production image size and CRC32.

IMAGE_BEGIN compatibility note: firmware accepts both the compact 12-byte form and the legacy 13-byte form. SmartDevice apps and PC tools should use the compact form so the complete ICD frame fits in a 20-byte ATT write when the negotiated MTU is 23.

SmartDevice sends IMAGE_DATA chunks using OFFSET:u16, BYTES:u8, DATA.

SmartDevice sends IMAGE_END and requires VERIFY=0.

SmartDevice sends REBOOT. Production firmware marks the staged slot valid before reset.

After reboot, device should advertise P531-Handheld if the production application is valid.

Failed update expectation: if production cannot run, the boot manager should fall back to the recovery application. The SmartDevice should scan for P531-Recovery, connect, use GET_INFO to confirm RECOVERY_MODE=1, then repeat IMAGE_BEGIN/DATA/END and REBOOT. Recovery GET_INFO returns the normal first 13 bytes plus MODE:u8, FLAGS:u16, UPTIME_MS:u32, RSSI:i8 fields.

## 7.1 Recovery Command Subset

| OPC | Name | Recovery support |
| --- | --- | --- |
| 0x01 | PING | Supported. |
| 0x02 | GET_INFO | Supported; 24-byte response includes recovery mode and flags. |
| 0x08 | REBOOT | Supported; commits pending verified image then resets. |
| 0x40 | IMAGE_BEGIN | Supported. |
| 0x41 | IMAGE_DATA | Supported. |
| 0x42 | IMAGE_END | Supported. |
| Other | - | Expected to return UNKNOWN_OPCODE. |

# 8. Recommended SmartDevice Flows

Connect to P531-Handheld, enable notifications, and send PING.

Send GET_INFO. If SERIAL=0 during production/calibration, provision with DEVICE_INFO_SET and verify with GET_INFO.

Send GET_STATUS and inspect PERIPH_PWR, SENSOR_FLAGS, FAULT_FLAGS, RSSI, and time state.

Optionally send SET_TIME so logs and measurement frames use real time.

Send SET_PERIPH_POWER=1 before one or more START_MEAS requests.

Use START_MEAS with MEAS_AVG in {1,2,4,8,16}; use 1 when no averaging is required.

Read LOG_GET when a command returns an unexpected error. Clear logs only as a deliberate service/manufacturing action.

Send SET_PERIPH_POWER=0 when measurement session is complete.

# 9. Security and Policy Notes

Administrative writes should be treated as secure operations by the SmartDevice/app policy: DEVICE_INFO_SET, CAL_SET, LED_CUR_SET, LOG_CLEAR, IMAGE_*, REBOOT, and ENTER_DEEP_SLEEP.

Manufacturing/calibration tools may allow rewriting identity and calibration values; production user flows should generally hide these controls.

No-image ICD regression covers normal command behavior. Full image-update regression should be run when IMAGE_* or boot/recovery code changes.

End of Document